PLEASE NOTE: The following specification contains areas, highlighted with the [ ] symbol. In these areas, the engineer has to make a selection, add specific, project related information and has to delete what is not applicable for the specific project.

ORP Sensor

Online process ORP sensor with built in temperature probe for measuring oxidation/reduction potential of water at high temperature. The sensor connects to a digital controller for operation.

Technical Data

Application	ORP at high temperature

Junction type	Flat, PTFE

Material	Reference Electrode: Ag/AgCl

Measuring range	-1500 mV - 1500 mV

Operating temperature range

0 - 110 °C

Pressure range	Max. 10 bar at 80 °C

Drift	<3 mV/week

Offset @ pH 7	N/A

Slope Range	N/A

Impedance	Reference impedance @ 25 °C: <50 kΩ

Glass impedance @ 25 °C: Platinum

Weight	0.62 kg

Scope of delivery

[ ] (delete what is not needed)

Sensor comes with a 10 m cable and manual

Accessories

[ ] PVC DN 40 flow chamber, flange fitting

[ ] PP immersion shaft, clip fitting (0.5, 1, 1.5 meter)

[ ] PP immersion shaft, adjustable flange (0.5, 1, 1.5 meter) [ ] PP immersion shaft, fixed flange (0.5, 1, 1.5 meter)

[ ] PP chemical cleaning kit (nozzle and feed pipes) [ ] Liquid reservoir unit

[ ] Junction box

[ ] Low-impedance cable, type 2666

[ ] High-impedance co-axial cable, type 2654

Services

[ ] Manufacturer’s services to perform start-up on instrument to include basic operational training and certification of

performance of the instrument.

[ ] Manufacturer’s Service Agreement that covers all the manufacturer’s recommended preventative maintenance, regularly scheduled calibration and any necessary repairs beginning from the time of equipment start-up through to end user acceptance / plant turnover and the first 12 months of end-user operation post turnover.

Brand:	Polymetron

Product:	8351 ORP Sensor